Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | ELE746 |
| Course Title | Power Systems Analysis |
| Semester/Year | F2025 |
| Instructor | Dr. Mohammadreza Arani |

| Lab/Tutorial Report NO. | Lab #1 |
| --- | --- |

| Report Title | Project 1: Two-Bus Single-Phase Distribution Feeder |
| --- | --- |

| Section No. | 02 |
| --- | --- |
| Submission Date | November 6, 2025 |
| Due Date | November 7, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Table of Contents

Pre-Lab:

- P1……………………………………………………………….2

- P2……………………………………………………………….2

- P3&4…………………………………………………………...3

Post-Lab:

- Introduction……………………………………………………..4

- Objectives…………………………………………………….…5

- Results………………………………………………………...…5

- Discussion………………………………………………………6

- Conclusion………………………………………………………6

- References………………………………………………………6

- Appendix……………………………………………………..….7

## Pre-Lab

#### P1

Given:

Vs =|Vs|∠0˚, Vs =|VL|∠δL, Z = jX

R = 0:

I = Vs - VLjX =|Vs|∠0˚ - |VL|∠δLjX

SL = PL + jQL  = VIL*

I* = |Vs|∠0˚ - |VL|∠δLjX

SL =  |VL|∠δL-jX(|Vs|∠0˚ - |VL|∠δL)

=  |Vs| - |VL|X (sinδL - j(1-cosδL))

Therefore:

PL =  |Vs| - |VL|X (sinδL)

QL  = |Vs| - |VL|X ((1-cosδL))

#### P2

Given:

RL = 0.9pu, X = 0.1pu, |Vs| = 1∠0°

ZL = RL + JXL

I = Vs/(Z + ZL)

VL = Vs - jXI

PL = |VL|2/RL, QL = -|VL|2/XL

XL = [−0.25,−0.10,0,+0.25]

| XL (pu) | kV | MVA (S) | MW (P) | Mvar (Q) | %PF | Amperes |
| --- | --- | --- | --- | --- | --- | --- |
| −0.25 | 73.3 | 1.268 | 1.222 | −0.339 | 96.35 | 9.755 |
| −0.10 | 73.3 | 1.240 | 1.255 | −0.136 | 96.30 | 9.829 |
| 0.00 | 73.3 | 1.240 | 1.240 | 0.000 | 100.00 | 9.829 |
| +0.25 | 73.3 | 1.132 | 1.091 | +0.303 | 96.35 | 9.217 |

Table 1 - Rated Load Values Used in ETAP for the Four XL Cases

#### Graphs (P3 and P4)

Figure 1 - MATLAB Graphs for  |VL|, δL, PL, and QL vs. XL

Figure 2 - MATLAB Graphs for  QL vs. δLand |Vs| - |VL|

Figure 3 - MATLAB Graphs for PL vs. δL and |Vs| - |VL|

## Post-Lab

#### Introduction

For the first lab experiment, the behavior of a two-bus single phase distribution feeder was observed. This is done to analyze the effects of load power factor system parameters on voltage regulation and power transfer. The study focused on the relationship between sending-end and receiving-end voltages, power transfer capabilities of the feeder, and the significance of reactive loading on system performance. Manual, MATLAB, and ETAP-based observation approaches were used to investigate the system response.

The distribution network studies is made of a voltage source supplying power through a feeder with an impedance Z = R + jX to a load ZL = RL + jXL. This circuit shows a radial distribution system which is similar to real-world networks.

#### Objectives

- To study the impact of the load power factor on voltage regulation in a distribution feeder.

- To analyze the effects of phase angle and magnitude variations between the sending-end and receiving-end voltages on real and reactive power transfer.

- To use MATLAB and ETAP software tools to simulate, visualize, and verify theoretical power system models.

#### Results

Figure 4 - ETAP Circuit Simulation Result

| Case | XL​ (pu) | Load (MVA) | Bus Voltage (kV) | Power Factor | Observations |
| --- | --- | --- | --- | --- | --- |
| 1 | -0.25 | 1.248 | 73.5 | Leading | Highest bus voltage; capacitive compensation improves regulation. |
| 2 | -0.10 | 1.24 | 73.3 | Slightly leading | Near unity PF; optimal real power transfer. |
| 3 | 0.00 | 1.24 | 73.1 | Unity | Voltage slightly lower; reference case. |
| 4 | +0.25 | 1.13 | 71.8 | Lagging | Significant voltage drop due to inductive loading. |

Table 2 - Observations and Results from ETAP Circuit

#### Discussion

Results from all three methods (manual, MATLAB, and ETAP) show that voltage regulation is very dependent on the load resistance XL. For capacitive loads (XL < 0), the receiving-end voltage rises and power transfer is improved. This is seen at XL = -0.25pu (approximately at 73.5kV). When XL is -0.10pu, both MATLAB and ETAP results present a near unity power factor and maximum real-power transfer. When XL is inductive (+0.25pu), the voltage drops (71.8kV) while apparent power decreases. Overall, voltage regulation worsens with lagging power factor and improves with capacitive compensation.

#### Conclusion

This experiment confirmed that a feeder’s voltage and power transfer depend on load power factor. A leading/capacitive power factor enhances voltage and efficiency while a lagging/inductive power factor does the opposite. Real power depends on phase-angle and the reactive power on voltage-magnitude differences. ETAP simulations match theoretical results which validate  the value of power-factor correction in distribution systems.

#### References

References

Toronto Metropolitan University. (2021). ELE 746 – Power System Analysis Fall 2025 Project 1: Two-Bus Single-Phase Distribution Feeder [PDF]. D2L. https://courses.torontomu.ca/d2l/le/content/1076725/viewContent/6668771/View

#### Appendix (Code)

% ELE746_Lab1_PreLab.m % Two-bus single-phase distribution feeder - pre-lab clear; clc; close all; % Given data Vs = 1; % sending-end voltage magnitude (pu) X = 0.1; % feeder reactance (pu) R = 0; % feeder resistance (pu) RL = 0.9; % load resistance (pu) XL_range = -0.25:0.05:0.25; % range of load reactance (pu) % Preallocate n = length(XL_range); VL_mag = zeros(1,n); deltaL_deg = zeros(1,n); PL = zeros(1,n); QL = zeros(1,n); for k = 1:n XL = XL_range(k); Z = 1j*X; ZL = RL + 1j*XL; I = Vs / (Z + ZL); % line current (phasor) VL = Vs - I*Z; % load voltage (phasor) VL_mag(k) = abs(VL); deltaL_deg(k) = angle(VL) * 180/pi; S_L = VL * conj(I); % load apparent power (pu) PL(k) = real(S_L); QL(k) = imag(S_L); end % Plot ¦V_L¦, deltaL, P_L, Q_L vs X_L figure('Name','ELE746 Lab1: Results vs X_L','NumberTitle','off','Units','centimeters','Position',[2 2 32 18]); subplot(2,2,1); plot(XL_range, VL_mag,'-o','LineWidth',1.4); grid on; xlabel('X_L (pu)'); ylabel('¦V_L¦ (pu)'); title('Load Voltage Magnitude'); subplot(2,2,2); plot(XL_range, deltaL_deg,'-o','LineWidth',1.4); grid on; xlabel('X_L (pu)'); ylabel('\delta_L (deg)'); title('Load Voltage Angle'); subplot(2,2,3); plot(XL_range, PL,'-o','LineWidth',1.4); grid on; xlabel('X_L (pu)'); ylabel('P_L (pu)'); title('Real Power at Load'); subplot(2,2,4); plot(XL_range, QL,'-o','LineWidth',1.4); grid on; xlabel('X_L (pu)'); ylabel('Q_L (pu)'); title('Reactive Power at Load'); % Compute voltage drop magnitude and negative delta dV = Vs - VL_mag; % ¦V_S¦ - ¦V_L¦ neg_delta = -deltaL_deg; % -delta_L in degrees % Plot P_L vs dV and -deltaL figure('Name','P_L vs dV and -deltaL','NumberTitle','off','Units','centimeters','Position',[2 2 28 10]); subplot(1,2,1); plot(dV, PL,'-o','LineWidth',1.4); grid on; xlabel('¦V_S¦ - ¦V_L¦ (pu)'); ylabel('P_L (pu)'); title('P_L vs ¦V_S¦-¦V_L¦'); subplot(1,2,2); plot(neg_delta, PL,'-o','LineWidth',1.4); grid on; xlabel('-\delta_L (deg)'); ylabel('P_L (pu)'); title('P_L vs -\delta_L'); % Plot Q_L vs dV and -deltaL figure('Name','Q_L vs dV and -deltaL','NumberTitle','off','Units','centimeters','Position',[2 2 28 10]); subplot(1,2,1); plot(dV, QL,'-o','LineWidth',1.4); grid on; xlabel('¦V_S¦ - ¦V_L¦ (pu)'); ylabel('Q_L (pu)'); title('Q_L vs ¦V_S¦-¦V_L¦'); subplot(1,2,2); plot(neg_delta, QL,'-o','LineWidth',1.4); grid on; xlabel('-\delta_L (deg)'); ylabel('Q_L (pu)'); title('Q_L vs -\delta_L');